LED BLINKING

ALGORITHM

- Start

- Initialize microcontroller (Arduino/ESP32/STM32 etc.) in MBED/Proteus.

- Configure Pin:

Set LED pin (e.g., P0 or D13) as OUTPUT.

- Loop Process:

Turn LED ON (write HIGH to pin).

Wait for delay (e.g., 1000 ms).

Turn LED OFF (write LOW to pin).

Wait for delay (e.g., 1000 ms).

- Repeat steps continuously (infinite loop).

- End

INPUT

void setup() {

pinMode(2, OUTPUT);

}

void loop() {

digitalWrite(2,HIGH);

delay(1000);

digitalWrite(2,LOW);

delay(1000);

}

Traffic light control system

Algorithm

- Start

- Initialize microcontroller (Arduino UNO/ESP32).

- Configure Pins:

Red LED → OUTPUT

Yellow LED → OUTPUT

Green LED → OUTPUT

- Loop Process:

Turn Red ON → wait 5 seconds.

Turn Red OFF, Green ON → wait 5 seconds.

Turn Green OFF, Yellow ON → wait 2 seconds.

Turn Yellow OFF.

- Repeat steps continuously (infinite loop).

- End

INPUT

int red = 3;

int yellow = 2;

int green = 1;

void setup() {

pinMode(red, OUTPUT);

pinMode(yellow, OUTPUT);

pinMode(green, OUTPUT);

}

void loop() {

// Red ON

digitalWrite(red, HIGH);

digitalWrite(yellow, LOW);

digitalWrite(green, LOW);

delay(3000);

// Yellow ON

digitalWrite(red, LOW);

digitalWrite(yellow, HIGH);

digitalWrite(green, LOW);

delay(2000);

// Green ON

digitalWrite(red, LOW);

digitalWrite(yellow, LOW);

digitalWrite(green, HIGH);

delay(3000);

}

OUTPUT

PIR MOTION ALERT SENSOR

ALGORITHM

- Start

- Initialize the PIR sensor, LED, and buzzer pins.

- Set the LED and buzzer as OUTPUT.

- Set the PIR sensor pin as INPUT.

- Read the PIR sensor output.

- Is motion detected?

  - Yes:

    - Turn LED ON.

    - Turn Buzzer ON.

    - Display/Print "Motion Detected!" (optional in Serial Monitor/LCD).

  - No:

    - Turn LED OFF.

    - Turn Buzzer OFF.

    - Display/Print "No Motion" (optional).

- Repeat Steps 5–6 continuously.

- End (program runs continuously in a loop).

Short Algorithm (for Record/Assignment)

- Start.

- Initialize PIR sensor, LED, and buzzer.

- Read PIR sensor output.

- If motion is detected:

  - LED = ON

  - Buzzer = ON

- Else:

  - LED = OFF

  - Buzzer = OFF

- Repeat the process continuously.

- End.

INPUT

const int pirPin = 2;

const int ledPin = 8;

const int buzzerPin = 9;

void setup() {

pinMode(pirPin, INPUT);

pinMode(ledPin, OUTPUT);

pinMode(buzzerPin, OUTPUT);

Serial.begin(9600);

}

void loop() {

int motion = digitalRead(pirPin);

if (motion == HIGH) {

digitalWrite(ledPin, HIGH);

digitalWrite(buzzerPin, HIGH);

Serial.println("Motion Detected!");

} else {

digitalWrite(ledPin, LOW);

digitalWrite(buzzerPin, LOW);

Serial.println("No Motion");

}

delay(100);

}

OUTPUT

DISTANCE MEASUREMENT USING ULTRASONIC SENSOR

ALGORITHM

      - Initialize the TRIG and ECHO pins.

      - Set TRIG as OUTPUT.

      - Set ECHO as INPUT.

      - Send a 10 µs trigger pulse through the TRIG pin.

      - Measure the echo pulse duration from the ECHO pin.

      - Calculate the distance using:
Distance = (Duration × 0.0343) / 2

      - Display the distance in the Serial Monitor.

      - Repeat Steps 5–8 continuously.

      - End (the program runs continuously in a loop).

INPUT

const int trigPin = 9;

const int echoPin = 10;

long duration;

float distance;

void setup() {

pinMode(trigPin, OUTPUT);

pinMode(echoPin, INPUT);

Serial.begin(9600);

}

void loop() {

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

duration = pulseIn(echoPin, HIGH);

distance = duration * 0.0343 / 2;

Serial.print("Distance: ");

Serial.print(distance);

Serial.println(" cm");

delay(500);

}

OUTPUT

TEMPARATURE ALERT USING DHT22 SENSOR

ALGORITHM

- Start.

- Initialize the DHT22 sensor, LED, and buzzer.

- Read the temperature.

- If temperature > 30°C:

LED = ON

Buzzer = ON

Else:

LED = OFF

Buzzer = OFF

- Repeat continuously.

- End.

INPUT

#include <DHT.h>

#define DHTPIN 2

#define DHTTYPE DHT22

DHT dht(DHTPIN, DHTTYPE);

void setup() {

Serial.begin(9600);

dht.begin();

}

void loop() {

float temperature = dht.readTemperature();

float humidity = dht.readHumidity();

if (isnan(temperature) || isnan(humidity)) {

Serial.println("Failed to read from DHT22 sensor!");

delay(2000);

return;

}

Serial.print("Temperature: ");

Serial.print(temperature);

Serial.println(" °C");

Serial.print("Humidity: ");

Serial.print(humidity);

Serial.println(" %");

Serial.println("----------------------");

delay(2000);

}

OUTPUT

11.07.2026

Sanitizer using ultrasonic distance sensor and servo motor

Algorithm;

  Start

  Initialize the Ultrasonic Sensor (HC-SR04) and Servo Motor.

  Set the servo motor to its initial position (0°).

  Send a trigger pulse from the ultrasonic sensor.

  Receive the echo signal and calculate the distance.

  Check if the measured distance is less than or equal to 10 cm.

  If Yes:

- Rotate the servo motor to 90° to press the sanitizer pump.

- Wait for 1 second.

- Rotate the servo motor back to 0°.

  If No:

- Keep the servo motor at 0°.

  Repeat the distance measurement continuously.

  End (the program keeps runnin

g in a loop).

Input;

#include <Servo.h>

Servo sanitizer;

const int trigPin = 9;

const int echoPin = 10;

const int servoPin = 6;

long duration;

int distance;

void setup()

{

pinMode(trigPin, OUTPUT);

pinMode(echoPin, INPUT);

sanitizer.attach(servoPin);

sanitizer.write(0);   // Initial position

Serial.begin(9600);

}

void loop()

{

// Trigger ultrasonic pulse

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

// Read echo

duration = pulseIn(echoPin, HIGH);

// Calculate distance

distance = duration * 0.034 / 2;

Serial.print("Distance: ");

Serial.print(distance);

Serial.println(" cm");

// Hand detected

if (distance > 0 && distance <= 10)

{

sanitizer.write(90);   // Press sanitizer pump

delay(2000);

sanitizer.write(0);    // Return to original position

delay(2000);

}

delay(100)

}

OUTPUT;

Pushbutton with buzzer          operation

Algorithm;

- Start.

- Initialize the push button pin as INPUT_PULLUP.

- Initialize the buzzer pin as OUTPUT.

- Read the state of the push button.

- If the push button is pressed:

  - Turn ON the buzzer.

- Otherwise:

  - Turn OFF the buzzer.

- Repeat the process continuously.

- Stop.

INPUT;

const int buttonPin = 2;

const int buzzerPin = 8;

void setup()

{

pinMode(buttonPin, INPUT_PULLUP);

pinMode(buzzerPin, OUTPUT);

}

void loop()

{

int buttonState = digitalRead(buttonPin);

if (buttonState == LOW)   // Button Pressed

{

digitalWrite(buzzerPin, HIGH);

}

else                      // Button Released

{

digitalWrite(buzzerPin, LOW);

}

OUTPUT;

TWO WAY LED WITH TWO PUSH BUTTON

ALGORITHM;

  Start.

  Initialize Push Button 1 (D2) and Push Button 2 (D3) as INPUT_PULLUP.

  Initialize the LED pin (D8) as OUTPUT.

  Read the status of Push Button 1 and Push Button 2.

  If Push Button 1 is pressed:

- Turn the LED ON.

  Else if Push Button 2 is pressed:

- Turn the LED OFF.

  Continue checking the button status repeatedly.

  Stop.

INPUT;

const int button1 = 10;

const int button2 = 11;

const int ledPin = 7;

void setup() {

pinMode(button1, INPUT_PULLUP);

pinMode(button2, INPUT_PULLUP);

pinMode(ledPin, OUTPUT);

digitalWrite(ledPin, LOW);

}

void loop() {

if (digitalRead(button1) == LOW) {

digitalWrite(ledPin, HIGH);   // Turn ON LED

}

if (digitalRead(button2) == LOW) {

digitalWrite(ledPin, LOW);    // Turn OFF LED

}

}

OUTPUT;